Finding the IP address once you are already connected to the pi:

hostname -I

(capital i) The ip address should be the first section

Startup, before running commands:

Log into raspberry pi and connect pi to Arduino using usb.

Location should be /dev/ttyUSB0 (might be 1 or 2 at the end)

If not, you may have to search through /dev/tty*

Run these commands on the pi:

stty -F /dev/ttyUSB2 9600 cs8 -cstopb -parenb

exec 3<>/dev/ttyUSB2

Any commands to the Arduino on the pi:

printf  ‘COMMAND\n’ >&3

e.g

printf  ‘ON\n’ >&3

Taking a photo and saving it:

Note: should be in home directory, ~ $, not Desktop

To take a photo run on the pi:

rpicam-still -n -o plant.jpg

Copy to computer (BE IN COMPUTER SHELL, NOT PI):

scp earth@<IP ADDRESS> :~/plant.jpg .

example:

scp earth@172.30.23.12:~/image.jpg .

This will save in whatever directory you are in

If you want to do a preset:

You can make a python program in nano.

Run to look at an example:

nano test.py

This uses the serial module, with serial.write() to send commands, serial.readline().decode().strip() to receive input from the Arduino.

Preset command:

In the Desktop/Python directory, run:

python3 preset.py

This will take a photo, but you will need to copy the image to computer. See above for instructions

Planned commands:

ARM_UP

ARM_DOWN

QUARTER_TURN_RIGHT

QUARTER_TURN_LEFT

RAISE_ARM
LOWER_ARM

ROBO_DANCE

EXIT